## Introduction

Assumptions:

- Compressor

  - Insulated system, Q = 0

  - Steady state and Steady flow

  - Isentropic efficiency

  - Change in Kinetic energy and Potential Energy zero

  - Win = 80 kW

  - Tin,comp = 21.2 C (294.2 K)

  - Cp air @ 1 bar = 1.006 kJ/kg

  - Cp air @ 50 bar = 1.127 kJ/kg

- Heat Exchanger

  - Insulated System, Q =0

  - Steady State and Steady Flow

  - Constant pressure

  - Change in kinetic and potential energy is zero

- Turbine

  - Insulated System

  - Turbine Efficiency, η = 0.85

  - Insulated system

  - Steady State and Steady Flow

  - Constant pressure

  - Change in kinetic and potential energy is zero

- Here insulated system is considered for analysis as there were multiple parameters for optimisation, could have made system complicated

## Activity 1:

This activity focuses on exergy balance of system which tells about the available energy which can be converted to useful work and exergy destroyed on the way to turbine in the form of irreversibility.

### Vessel Parameters and Assumptions

Starting with defining vessel assumption,

Vvessel = 100 m3, Vmax,vessel = 100 m3

Pmax = 50 bar , Pmin = 2 bar

Tmax = 500 C = 773K

### Based on provided parameters:

### Mass Calculation

Starting calculation by applying ideal gas equation 1

PV=MR0T 						(1)

Maximum mass, Mmax = 2253.76491 kg air

Minimum mass, Mmin = 90.150 kg air

### Mass Flow Rate Calculation

Compressor Mass Flow Rate:

mcompressor= Mmax- Mmintcomp=0.04292 kg/s

(Required to fill the vessel over time frame of 14 hour)

mturbine= Mmax- Mmin tturbine=0.1502 kg/s

Note: Minimum pressure of 2 bar is taken so that turbine has sufficient pressure gradient to drive turbine, reference taken from outlet pressure of 1.1 bar

## Compressor Availability Analysis

For compressor Win = 80 kW which is considered as negative as it is work input, using first law of thermodynamics

Q-W=  mair*hin- hout

h= Cp*T

Tout, compressor = 1916.16 K was calculated

For activity 1 availability analysis is done on individual component to calculate exergy available, equations used are given below

B1+ Qin 1- T0T= B1+W+ ∆AE                                       (2)

B1-B2=mair h1 -h2- T0mairs1-s2                                  (3)

s2 -s1=Cp,air lnT2T1 - R0,air lnP2P1                                          (4)

After using equations above B1 – B2 = -67.508 kW

In compressor we add energy in air so B1 – B2 is negative indicating the exergy at outlet is more than that of exergy at inlet which means fluid is charged with energy

Some amount of exergy is destroyed in the process due to internal irreversibility such as friction, turbulence which is represented by ∆AE.

∆AE= Win- Wrev=12.4 kW

For compressor we can conclude that the amount of available energy after compression is 67 kW, but energy destroyed in high which is 12.4 kW so this component need optimisation.

## Heat Exchanger Availability Analysis

Heat exchanger here is used to boil water until it reaches 80 C, the hot air stream enters heat exchanger at Tin = 1916.1611 K, and exits at T,out = 773 K

For heat exchanger the availability analysis is done using equation

BH,1+ BC,1= BH,2+ BC,2+ ∆AE                                                          (5)

B1 – B2 = ∆AE = 37.44665 kW

In heat exchangers, exergy destroyed is equivalent to available exergy in system. This total available exergy in system is utilized to boil 7489.3315 kg of water. The available energy in heat exchanger can be optimised depending on the quantity of water required. Key parameters for optimising the design are flow rate of air stream, system maximum pressure, and vessel minimum pressure.

## Turbine Availability Analysis

In our system turbine runs for the period of 4 hours with 85% efficiency, temperature of air entering system is 773 K, and after expanding temperature of air is 336 K at pressure of 50 bar

Using first law of thermodynamics, the work produced by turbine is 73.87 kW.

∆AE= Wideal- Wreal=13.04 kW

## Conclusion after Activity 1

After activity 1 the total efficiency of system including thermal boiling was found to be 77.8 % which is pretty good for compressed air energy storage, but amount of exergy destroyed is pretty high in compressor and turbine which needs to be reduced by proper parameter setting to get required amount of Q from heat exchanger and power output form turbine.

Activity 2:

Before system design, optimisation is carried out to increase efficiency of system and model design according to requirement

Requirement:

Design a system for an area which has approx. 20-30 houses, CAES power will be taken from solar panel

Assumption:

- Assuming that electricity required for the period of 14 hour is provided by solar and rest for the period of 10 hour electricity is extracted from turbine

- Designing system in such a way that water which is heated during the period of 14 hour is used for whole day, Assuming that each house require around 120-150 L of hot water per day (shower + washing clothes + dishwasher)

- So, no electricity will go towards compressor when turbine runs

- Turbine runs for the period of 4 hour

- For this activity I am changing values of Pmax , Pmin  and Tair,vessel is kept constant

Now to optimise system we have to differentiate dependent variable, as all design parameters change with change in system maximum pressure, vessel minimum pressure and vessel size. In order to design a system which is optimal for providing energy for 25 houses over a period of 24 hour, we will analyse all variables with respect to the given three specific parameters

## Impact of Maximum pressure

To analyse effects of maximum pressure, the operating range considered is between 50-100 bar with increments of 10 bar. The system variables which are affected with change are:

- Mass of vessel: Maximum air mass within storage vessel

- Flow rates: Mass flow rate for turbine and compressor

- Thermal performance: Compressor outlet temperature and heat exchanger duty

- System Output: Turbine power output and cycle efficiency

### Optimisation of compressor

The optimisation of compressor is governed by compressor outlet temperature, mass flow rate as this are dependent on mass of vessel along with time period compressor runs for that is 14 hour.

This factors also affect exergy that is available at outlet and exergy that is destroyed inside the compressor

To show change in dependent variable I have optimised system in excel sheet which can be referred. Here I am only explaining graphs and why this changes happen

- Pmax vs Maximum mass of vessel (Mmax)

| P,max (bar) | M,max (kg of air) |
| --- | --- |
| 50 | 2253.764914 |
| 60 | 2704.517897 |
| 70 | 3155.27088 |
| 80 | 3606.023863 |
| 90 | 4056.776846 |
| 100 | 4507.529829 |

Figure 1 P,max vs M,max

The linear relationship between maximum mass of vessel and maximum pressure is shown in Figure 1, depending on ideal gas law. But for maintaining the system as pressure increases it requires high CAPEX and OPEX, and it is not a viable option to design a very high pressure system just to power 20-30 houses.

- Pmax vs mass flow rate compressor (𝑚̇ 𝑐𝑜𝑚𝑝)

| P,max (bar) | m,air comp (kg/s) |
| --- | --- |
| 50 | 0.042928856 |
| 60 | 0.051872367 |
| 70 | 0.060815879 |
| 80 | 0.06975939 |
| 90 | 0.078702902 |
| 100 | 0.087646413 |

Figure 2 Maximum pressure vs mass flow rate compressor

The graph shows linear relationship between maximum pressure and mass flow rate compressor, this is because the mass of vessel changes with pressure and amount of time for filling of vessel is 14 hour which stays same, so mass flow rate increases which in turn affect compressor outlet temperature

- Pmax vs Temperature outlet compressor

| P,max (bar) | T out,comp (K) |
| --- | --- |
| 50 | 1916.161136 |
| 60 | 1631.066683 |
| 70 | 1429.82354 |
| 80 | 1280.181203 |
| 90 | 1164.548488 |
| 100 | 1072.514286 |

As shown in figure 3, Outlet temperature of compressor inversely changes with maximum pressure, it is due to change in mass flow rate of air in compressor as the speed is increasing residence time of air in each stage decreases leading to inverse relationship which affects exergy balance and destruction potentially affecting efficiency

Figure 3 Maximum pressure vs Temperature outlet compressor

- Pmax vs Available exergy (B1 – B2)

| P,max (bar) | B1 - B2 (comp) |
| --- | --- |
| 50 | -67.55965551 |
| 60 | -68.52247879 |
| 70 | -69.9764038 |
| 80 | -71.83219967 |
| 90 | -74.02484831 |
| 100 | -76.50510398 |

As shown in Figure 4 graph, the magnitude of available energy in compressor is increasing with pressure, thus as pressure increases the exergy available at output increases

The negative sign value of B1 –B2 represent that exergy at output of compressor is more than exergy at input

Figure 4 Maximum pressure vs Available energy

- Pmax vs Exergy destruction in compressor

| P,max (bar) | ∆AE kW |
| --- | --- |
| 50 | 12.44034449 |
| 60 | 11.47752121 |
| 70 | 10.0235962 |
| 80 | 8.167800333 |
| 90 | 5.975151689 |
| 100 | 3.494896023 |

Figure 5 Maximum pressure vs Exergy destroyed

As shown in figure 5 graph, the magnitude of exergy destruction within the compressor is decreasing as increase in pressure, this is related to decrease in air temperature and air residence time in stages as pressure increases

### Optimisation of Heat Exchanger

Change in maximum pressure results in different air inlet temperature in heat exchanger, which in turn affect the total heat duty. Other factors which are affected when optimising heat exchanger are mass flow rate air, water boiled during period of 14 hour and temperature difference between air and water stream as it will define the design of heat exchanger

1. Pmax vs Mass flow rate air heat exchanger

| P,max (bar) | m,air comp (kg/s) |
| --- | --- |
| 50 | 0.042928856 |
| 60 | 0.051872367 |
| 70 | 0.060815879 |
| 80 | 0.06975939 |
| 90 | 0.078702902 |
| 100 | 0.087646413 |

As it is shown in figure 2, with increase in maximum pressure mass flow rate in heat exchanger also decreases like compressor, which affects heat duty of exchanger along with water boiled during period of 14 hour

Note: Pmax vs Mass flow rate compressor graph is same for every component in system

Figure 6 Maximum pressure vs Mass flow rate water heat exchanger

2. Pmax vs Mass flow rate water

| P,max (bar) | mass flow rate, water (kg/s) |
| --- | --- |
| 50 | 0.219472513 |
| 60 | 0.199058387 |
| 70 | 0.178644261 |
| 80 | 0.158230136 |
| 90 | 0.13781601 |
| 100 | 0.117401884 |

As shown in figure 6, mass flow rate of water decreases with increase in pressure this is because mass flow rate of air increases, leading to lower heat duty and results in decrease of mass flow rate of water

Figure 7 Maximum pressure vs water boiled

3. Pmax vs water boiled

| P,max (bar) | kg of water boiled in 14 hour |
| --- | --- |
| 50 | 11061.41466 |
| 60 | 10032.54272 |
| 70 | 9003.670775 |
| 80 | 7974.79883 |
| 90 | 6945.926885 |
| 100 | 5917.05494 |

As shown in figure 7, the amount of water boiled during the period of 14 hour decreases with increase in pressure, as heat duty of exchanger decreases it affects the amount of water boiled

4.  Pmax vs ∆AE and B1-B2

| P,max (bar) | ∆AE, kW | B1-B2,kW |
| --- | --- | --- |
| 50 | 37.44661823 | 37.44661823 |
| 60 | 32.84489274 | 32.84489274 |
| 70 | 28.60410004 | 28.60410004 |
| 80 | 24.65426992 | 24.65426992 |
| 90 | 20.94466885 | 20.94466885 |
| 100 | 17.43707028 | 17.43707028 |

Figure 8 Maximum pressure vs ∆AE and B1 –B2

If we consider the case of heat exchanger the amount of exergy destroyed is equal to amount of exergy available in system and as pressure increases the exergy available in system decreases.

### Optimisation of Turbine

Change in maximum pressure affects mass flow rate of air, power output and exergy destroyed in turbine. Turbine is designed keeping in mind that sometime while expanding the air cools and freezes

1. Pmax vs mass flow rate air in turbine

| P,max (bar) | m, air , turbine (kg/s) |
| --- | --- |
| 50 | 0.150250994 |
| 60 | 0.181553285 |
| 70 | 0.212855575 |
| 80 | 0.244157866 |
| 90 | 0.275460156 |
| 100 | 0.306762447 |

Mass flow rate of air in turbine depends upon maximum and minimum size of vessel, which changes with pressure of system. As shown in figure 9, the maximum pressure increase leads to increase in mass flow rate of turbine

Figure 9 Maximum pressure vs Mass flow rate turbine

2. Pmax vs Woutput

| P,max (bar) | W,real kW |
| --- | --- |
| 50 | 73.8709628 |
| 60 | 89.26074672 |
| 70 | 104.6505306 |
| 80 | 120.0403146 |
| 90 | 135.4300985 |
| 100 | 150.8198824 |

Figure 10 Maximum pressure vs Power Output turbine

As shown in figure 10, the linear relationship between maximum pressure and power output increases, this is due to mass flow rate of air in system expands to generate specified power output.

3. Pmax vs ∆AE (Availability Destruction)

| P,max (bar) | ∆AE kW |
| --- | --- |
| 50 | 13.03605226 |
| 60 | 15.75189648 |
| 70 | 18.4677407 |
| 80 | 21.18358492 |
| 90 | 23.89942914 |
| 100 | 26.61527336 |

Figure 11 Maximum pressure vs Availability Destruction

As shown in figure 11, availability destruction increases as maximum pressure rises, the turbine operates at efficiency of 85% as mass flow rate increases the power output increases, when compared with isentropic turbine the power output also increases with increase in pressure which leads to more exergy destruction

### Overall efficiency of System

The calculated efficiency is combined efficiency for heat used to boil water and work output by turbine. This value provides overview of system work we have to keep in mind that heat and electricity as work output cannot be directly compared as electricity as work output is more valuable that heat.

1. Pmax vs Efficiency of system

| P,max (bar) | Efficiency of system |
| --- | --- |
| 50 | 0.955163284 |
| 60 | 0.945822301 |
| 70 | 0.936481319 |
| 80 | 0.927140336 |
| 90 | 0.917799353 |
| 100 | 0.908458371 |

Figure 12 Maximum pressure vs Efficiency of system

As we increase maximum pressure the overall efficiency of system decreases, this is due to the fact that mass flow rate, heat exchanger duty, turbine power output all vary with pressure. Ultimately, the design of system entirely depends on your requirements as efficiency here is function of both heat duty in heat exchanger as well as power output in turbine. In this context the efficiency is just a way to quantify how much work can produced with change in pressure

## Impact of change in Minimum pressure

In this section we analyze how change in minimum pressure vessel affects the system performance parameters. The general trend in change align to changes we saw in maximum pressure analysis, for minimum pressure analysis I will provide relevant graphical data and focus on result which are different from previously discussed maximum pressure analysis

### Optimisation of compressor

1. Pmin vs minimum mass of vessel

| P,min (bar) | M,min (kg) |
| --- | --- |
| 2 | 90.15059657 |
| 3 | 135.2258949 |
| 4 | 180.3011931 |
| 5 | 225.3764914 |
| 6 | 270.4517897 |
| 7 | 315.527088 |
| 8 | 360.6023863 |
| 9 | 405.6776846 |
| 10 | 450.7529829 |

Figure 13 Minimum pressure in Vessel vs Minimum mass of air vessel

As show in figure 13, the minimum mass of air in vessel and minimum pressure shares linear relationship, as we increase minimum pressure in vessel the minimum mass increases as per first law of thermodynamics

2. Pmin vs Mass flow rate compressor

| P,min (bar) | mass flow rate compressor (kg/s) |
| --- | --- |
| 2 | 0.042928856 |
| 3 | 0.042034504 |
| 4 | 0.041140153 |
| 5 | 0.040245802 |
| 6 | 0.039351451 |
| 7 | 0.0384571 |
| 8 | 0.037562749 |
| 9 | 0.036668397 |
| 10 | 0.035774046 |

Figure 14 Minimum pressure in vessel vs Mass flow rate compressor

As shown in figure 14, the compressor mass flow rate decreases as minimum pressure of vessel increases, the rise in minimum pressure increases the minimum air mass in vessel which in turn affects the mass flow rate as there is less capacity to fill for given time period of 14 hour

3. Pmin vs Tout compressor

| P,min (bar) | T out,comp (K) |
| --- | --- |
| 2 | 1916.161136 |
| 3 | 1951.343004 |
| 4 | 1988.05452 |
| 5 | 2026.397657 |
| 6 | 2066.483665 |
| 7 | 2108.434139 |
| 8 | 2152.382254 |
| 9 | 2198.474179 |
| 10 | 2246.870701 |

Figure 15 Minimum pressure vessel vs Temperature outlet compressor

As shown in figure 15, the compressor outlet temperature increases as we go on increasing minimum pressure in vessel, this variation is related to compressor’s mass flow rate decrease, this change increases the residence time of air at each stage which leads to increase in temperature of air outlet in compressor

4. Pmin vs change in availability (B1-B2)

| P,min (bar) | B1 - B2 (comp) |
| --- | --- |
| 2 | -67.55965551 |
| 3 | -67.56629088 |
| 4 | -67.57763275 |
| 5 | -67.59381111 |
| 6 | -67.61496197 |
| 7 | -67.6412278 |
| 8 | -67.67275794 |
| 9 | -67.7097091 |
| 10 | -67.75224595 |

Figure 16 Minimum pressure vs Availability analysis

As shown in figure 16, change is availability with increase in minimum pressure vessel decreases but change is negligible.

5. Pmin vs Exergy destruction

| P,min (bar) | ∆AE kW |
| --- | --- |
| 2 | 12.44034449 |
| 3 | 12.43370912 |
| 4 | 12.42236725 |
| 5 | 12.40618889 |
| 6 | 12.38503803 |
| 7 | 12.3587722 |
| 8 | 12.32724206 |
| 9 | 12.2902909 |
| 10 | 12.24775405 |

Figure 17 Minimum pressure vs Exergy destruction

As shown in figure 17, destruction in exergy decreases with increase in Minimum pressure but change is negligible

### Optimisation of heat exchanger

The change in minimum pressure leads to change in mass flow rate of air, temperature inlet, mass flow rate water, heat duty and amount of water boiled.

1. Pmin vs mass flow rate of air heat exchanger

| P,min (bar) | mass flow rate air in heat exchanger (kg/s) |
| --- | --- |
| 2 | 0.042928856 |
| 3 | 0.042034504 |
| 4 | 0.041140153 |
| 5 | 0.040245802 |
| 6 | 0.039351451 |
| 7 | 0.0384571 |
| 8 | 0.037562749 |
| 9 | 0.036668397 |
| 10 | 0.035774046 |

As shown in figure 14, mass flow rate of air decreases with increase in minimum pressure of vessel

2. Pmin vs Temperature air inlet

| P,min (bar) | T in,air heat exchanger (K) |
| --- | --- |
| 2 | 1916.161136 |
| 3 | 1951.343004 |
| 4 | 1988.05452 |
| 5 | 2026.397657 |
| 6 | 2066.483665 |
| 7 | 2108.434139 |
| 8 | 2152.382254 |
| 9 | 2198.474179 |
| 10 | 2246.870701 |

As shown in figure 18, temperature of air increases with increase in minimum pressure it is due to change in mass flow rate in compressor

Figure 18 Minimum pressure vessel vs Temperature air heat exchanger

Figure 19 Vessel minimum pressure vs Water boiled

3. Pmin vs Mass flow rate water

| P,min (bar) | mass flow rate water, (kg/s) |
| --- | --- |
| 2 | 0.219472513 |
| 3 | 0.221513926 |
| 4 | 0.223555338 |
| 5 | 0.225596751 |
| 6 | 0.227638164 |
| 7 | 0.229679576 |
| 8 | 0.231720989 |
| 9 | 0.233762401 |
| 10 | 0.235803814 |

As shown in figure 19, the amount of water boiled increases as we raise minimum pressure vessel, this trend is function of mass flow rate of water and heat duty of heat exchanger. The heat duty of exchanger increases as mass flow rate of air decreases so the amount of water that is boiled in given time frame increases

4. Pmin vs exergy destruction and change in available exergy

| P,min (bar) | ∆AE, kW |
| --- | --- |
| 2 | 37.44661823 |
| 3 | 37.93017982 |
| 4 | 38.41844792 |
| 5 | 38.9115525 |
| 6 | 39.4096296 |
| 7 | 39.91282166 |
| 8 | 40.42127802 |
| 9 | 40.93515541 |
| 10 | 41.45461848 |

Figure 20 Minimum pressure vs Exergy destruction

As shown in figure 20, the destruction of availability increases with increase in minimum pressure. As previously discussed, heat duty in exchanger increases as we increase minimum mass of vessel and in heat exchanger configuration the amount of exergy destroyed is equal to amount of exergy available in the system

### Optimisation of turbine

Change in minimum pressure vessel affects mass flow rate of air, power output and exergy destroyed in turbine.

1. Pmin vs Mass flow rate of air turbine

| P,min (bar) | m, air , turbine (kg/s) |
| --- | --- |
| 2 | 0.150250994 |
| 3 | 0.147120765 |
| 4 | 0.143990536 |
| 5 | 0.140860307 |
| 6 | 0.137730078 |
| 7 | 0.134599849 |
| 8 | 0.13146962 |
| 9 | 0.128339391 |
| 10 | 0.125209162 |

Figure 21 Minimum pressure vessel vs Mass flow rate turbine

As shown in figure 21, the mass flow rate of turbine decreases as we increase minimum pressure vessel, as mass flow rate depends upon difference between maximum and minimum mass of vessel which decreases as we increase minimum pressure leading to decrease in mass flow rate of air in turbine

2.  Pmin vs W output and exergy destruction

| P,min (bar) | W,real kW | ∆AE kW |
| --- | --- | --- |
| 2 | 73.8709628 | 13.03605226 |
| 3 | 72.33198441 | 12.76446784 |
| 4 | 70.79300602 | 12.49288341 |
| 5 | 69.25402763 | 12.22129899 |
| 6 | 67.71504923 | 11.94971457 |
| 7 | 66.17607084 | 11.67813015 |
| 8 | 64.63709245 | 11.40654573 |
| 9 | 63.09811406 | 11.1349613 |
| 10 | 61.55913567 | 10.86337688 |

Figure 22 Minimum pressure vessel vs work output and exergy destruction

As shown in figure 21, work output and exergy destruction both decreases as we increase minimum pressure vessel. This can be due to mass flow rate of turbine decreases, this leads to decrease in ideal and real work output, which in turn drops the amount of exergy destroyed

### Efficiency of system

1. Pmin vs Efficiency of system

| P,vessel,min (bar) | Efficiency of system |
| --- | --- |
| 2 | 0.955163284 |
| 3 | 0.956097382 |
| 4 | 0.95703148 |
| 5 | 0.957965578 |
| 6 | 0.958899677 |
| 7 | 0.959833775 |
| 8 | 0.960767873 |
| 9 | 0.961701971 |
| 10 | 0.96263607 |

Figure 23 Minimum vessel pressure vs Efficiency of System

As we increase minimum pressure the overall efficiency of system increases, this improvement is due to mass flow rate, heat exchanger duty, turbine power output all vary with pressure. Ultimately, the design of system entirely depends on your requirements as efficiency here is function of both heat duty in heat exchanger as well as power output in turbine. And efficiency increases even when turbine power output decreases because heat exchanger duty increases affecting overall efficiency of the system

## Impact of change in Vessel Size

In this section we analyze how change in vessel size affects the system performance parameters. The general trend in change align to changes we saw in earlier analysis, for vessel size analysis I will provide relevant graphical data and focus on result which are different from previously discussed analysis

1. Vessel size vs maximum mass of air in vessel

| Vessel Size (m^3) | M,max kg air | M,min kg air |
| --- | --- | --- |
| 100 | 2253.764914 | 90.15059657 |
| 140 | 3155.27088 | 126.2108352 |
| 180 | 4056.776846 | 162.2710738 |
| 220 | 4958.282811 | 198.3313125 |
| 260 | 5859.788777 | 234.3915511 |
| 300 | 6761.294743 | 270.4517897 |

Figure 24 Maximum vessel size vs Mass of air

As shown in figure 24, the maximum and minimum mass of air in vessel increases as we increase the size of vessel keeping pressure constant this is because maximum volume of vessel is increasing leading to increase in mass when ideal gas law is applied

### Optimisation of Compressor

1. Vessel size vs mass flow rate compressor

| Vessel Size (m^3) | mass flow rate compressor, (kg/s) |
| --- | --- |
| 100 | 0.042928856 |
| 140 | 0.060100398 |
| 180 | 0.07727194 |
| 220 | 0.094443482 |
| 260 | 0.111615024 |
| 300 | 0.128786567 |

Figure 25 Vessel size vs mass flow rate compressor

As shown in figure 25, increasing vessel volume leads to increase in mass flow rate of air in compressor, as mass flow rate is function of difference in maximum and minimum mass of air and time is kept constant so it increases to fill larger capacity

2. Vessel size vs Temperature outlet compressor

| Vessel Size (m^3) | T out,comp (K) |
| --- | --- |
| 100 | 1916.161136 |
| 140 | 1443.7189 |
| 180 | 1181.250991 |
| 220 | 1014.225958 |
| 260 | 898.5932428 |
| 300 | 813.7959184 |

Figure 26 Vessel size vs Temperature outlet compressor

As shown in figure 26, the temperature of compressor outlet decreases as we increase the vessel volume keeping pressure constant. The reason is because mass flow rate of air in compressor increases leading to drop in temperature as residence time of air decreases at each stage

3. Vessel size vs change in availability

| Vessel Size (m^3) | B1 - B2 (comp) |
| --- | --- |
| 100 | -67.55965551 |
| 140 | -68.20189545 |
| 180 | -69.95075978 |
| 220 | -72.47189077 |
| 260 | -75.56464322 |
| 300 | -79.09756764 |

Figure 27 Vessel Size vs Change in availability

As shown in figure 27, change in availability increases as we increase vessel size, negative sign shows that outlet of compressor has more exergy than inlet

4. Vessel size vs exergy destruction

| Vessel Size (m^3) | ∆AE kW |
| --- | --- |
| 100 | 12.44034449 |
| 140 | 11.79810455 |
| 180 | 10.04924022 |
| 220 | 7.528109226 |
| 260 | 4.435356781 |
| 300 | 0.902432363 |

Figure 28 Vessel Size vs Availability destruction

As shown in figure 28, exergy destruction in compressor reduces as we increase vessel size this is because mass flow rate of air increases due to which at each compression stage air spends less time leading to less energy wasted due to friction and turbulence

### Optimisation of Heat exchanger

The change in vessel size leads to change in mass flow rate of air, temperature inlet, mass flow rate water, heat duty and amount of water boiled as we have analysed earlier

1. Vessel size vs mass flow rate heat exchanger

| Vessel Size (m^3) | mass flow rate air in heat exchanger (kg/s) |
| --- | --- |
| 100 | 0.042928856 |
| 140 | 0.060100398 |
| 180 | 0.07727194 |
| 220 | 0.094443482 |
| 260 | 0.111615024 |
| 300 | 0.128786567 |

For graph you can see figure 25, Mass flow rate is same for heat exchanger as compressor

2. Vessel size vs Temperature air inlet

| Vessel Size (m^3) | T in,air heat exchanger (K) |
| --- | --- |
| 100 | 1916.161136 |
| 140 | 1443.7189 |
| 180 | 1181.250991 |
| 220 | 1014.225958 |
| 260 | 898.5932428 |
| 300 | 813.7959184 |

Figure 29 Vessel size vs temperature inlet heat exchanger

As shown in figure 29, the air inlet temperature decreases with increase In vessel size as discussed earlier

3. Vessel size vs mass flow rate water

| Vessel Size (m^3) | mass flow rate water, (kg/s) |
| --- | --- |
| 100 | 0.219472513 |
| 140 | 0.180277391 |
| 180 | 0.14108227 |
| 220 | 0.101887148 |
| 260 | 0.062692026 |
| 300 | 0.023496905 |

Figure 30 Vessel size vs mass flow rate water

As shown in figure 30, the mass flow rate of water decreases with increase in vessel size, this relation is due to decrease in heat duty in exchanger which reduce mass flow rate of water in given time

4. Vessel size vs Exergy destruction and change in availability

| Vessel Size (m^3) | ∆AE, kW |
| --- | --- |
| 100 | 37.44661823 |
| 140 | 28.93187458 |
| 180 | 21.52375534 |
| 220 | 14.88790276 |
| 260 | 8.823671631 |
| 300 | 3.199612473 |

Figure 31 Vessel size vs Exergy destruction

As shown in figure 31, as we increase vessel size the exergy destruction and exergy available decreases in heat exchanger due to increasing of mass flow rate of air

5. Vessel size vs Water boiled

| Vessel Size (m^3) | Amount of water boiled |
| --- | --- |
| 100 | 11061.41466 |
| 140 | 9085.98053 |
| 180 | 7110.546396 |
| 220 | 5135.112262 |
| 260 | 3159.678128 |
| 300 | 1184.243994 |

Figure 32 Vessel size vs Amount of water boiled

As shown in figure 32, the amount of water boiled decreases with increase in vessel size as heat duty in exchanger decreases leading to less amount of water boiled in given time frame of hour

### Optimisation of Turbine

1. Vessel size vs Mass flow rate turbine

| Vessel Size (m^3) | m, air , turbine (kg/s) |
| --- | --- |
| 100 | 0.150250994 |
| 140 | 0.210351392 |
| 180 | 0.27045179 |
| 220 | 0.330552187 |
| 260 | 0.390652585 |
| 300 | 0.450752983 |

Figure 33 Vessel size vs Mass flow rate turbine

As shown in figure 33, as we increase vessel size mass flow rate in turbine increases as difference between maximum and minimum vessel size increases and mass is function of time which is kept constant to 14 hour so mass flow rate of turbine increases

2. Vessel Size vs W,real

| Vessel Size (m^3) | W,real kW |
| --- | --- |
| 100 | 73.8709628 |
| 140 | 103.4193479 |
| 180 | 132.967733 |
| 220 | 162.5161182 |
| 260 | 192.0645033 |
| 300 | 221.6128884 |

Figure 34 Vessel size vs Turbine Work output

As shown in figure 34, as we increase vessel size the amount of power output and exergy destruction increases. This is because larger vessel increases total mass of air which results to more power output and exergy destruction

### Efficiency of system

| Vessel size (m^3) | Efficiency of system |
| --- | --- |
| 100 | 0.955163284 |
| 140 | 0.937228597 |
| 180 | 0.919293911 |
| 220 | 0.901359224 |
| 260 | 0.883424538 |
| 300 | 0.865489851 |

Figure 35 Vessel size vs Efficiency of system

As shown in figure 35, as we increase vessel size the efficiency of system decreases it is dependent on multiple factors. The pressure is kept constant, mass flow rate of air in turbine and compressor, heat exchanger duty. Even though the power output in turbine is increasing thermal output of heat exchanger is decreasing leading to less efficiency of system. But we have to note that electricity as power is more valuable than heat, and it totally depends on what you are expecting from system. Efficiency is just a way to quantify power output of the system.

## Design of System by taking optimisation in consideration

When we look into graphs above and make sense of numbers what are shown, they are for ideal case scenario, but in real world maintaining a high pressure system is costly and require specific material for construction which leads to high OPEX and CAPEX.

The design of system in my case is for the row of 25 houses which need supply for hot water and electricity for night, during day time electricity is provided by solar panel and hot water by CAES system. When solar panel does not work, electricity is given by turbine and water is already heated during morning and stored for use.

I selected row of houses because when used maximum vessel size that is 300 m^3, due to pressure, mass flow rate, temperature outlet compressor I was not getting an efficient system for both use.
In period of 10 hour each house needs around 2.4 kWh of electricity and 120-150 L of hot water for period of 24 hour.

If we consider energy output of our system it provides 3317. 638 kg of hot water daily which averages to around 132 kg of hot water for each house, as each day is not same so amount of water will be adjusted accordingly. And turbine provides 616 kWh of electricity for period of 4 hour, which will be used by 25 houses for period of 10 hour

(For detailed calculation take reference from excel sheet)

## Conclusion

For the given CAES system, activity one was all about setting up variables and doing thermodynamic analysis for the system. So that we can define variables for activity 2, for activity 2 our main focus was to optimise the design such that it can be used in real world scenario. I used given pressure and volume for system which is essentially for row of houses, but can be changed with requirement. I did not change the time turbine runs for, as optimisation done is for period of 4 hour in each case, if no. of houses is increased the time turbine runs for can be decreased to get more electricity output and for water output pressure of compressor can be decreased to get more amount of water, and once system is constructed you cannot change vessel easily so considered a bigger size of vessel

- Sizing and Pressure Management

Maintaining a high pressure system requires high CAPEX and OPEX, and special material like high grade steel is required

- Time constraint

A better system could have been designed if running time of turbine was increased, as electricity is used for the period of 10 hour and turbine discharge duration is 4 hour

The efficiency of designed system is 75% which aligns with real world scenario