Heat Exchangers and Heat Transfer (2025-2026)

Course code: B51GL

Assignment 2

Radiation

Name: Gauravsingh Rajput (H 00515081)

Radiation/ Convection Network Assessment – System HB

Your thermal system is a small cylindrical furnace.   The objects being heated up are cylindrical billets of steel (s=8000 kg/m3 and cs=420 J/kg K), see the figure below.

Comb Prods

Fuel

Air

Surface 1

Surface 3

Surface 2

2m

Surface 4

Surface 5

1m

2m

0.5m

Steel Billet

Figure HB – Furnace dimensions.

This is essentially a technical calculation / analysis of a heat transfer system.  There must be sufficient discussion to make it straight forward for the marker to assess a student’s understanding.

Mixed-mode heat transfer

In this section, both radiation heat transfer and convection heat transfer will be considered together.  The surface properties are summarised in the table below.

Number of Surfaces=5

| Surface | Temperature / K |  /W m-2K-1 | Emissivity |
| --- | --- | --- | --- |
| 1 | 1200 | 20 | 0.8 |
| 2 | 1400 | 15 | 0.8 |
| 3 | 1600 | 20 | 1 |
| 4 | 293 | 15 | 0.5 |
| 5 | 293 | 10 | 0.5 |

Produce a resistance network to include convection between the surfaces and the air in the trough surrounding the steel billet (label the temperature in the trough as Tgas).

Assumptions:

- Calculate the temperature of the air in the space surrounding the pipe (Tgas).

- Calculate the net radiation heat flux, convective heat flux and total heat flux to all surfaces.

Assuming the net heat fluxes do not change significantly estimate the time it takes for the water in the pipe (hot water system) or steel billet (furnace system) to increase from 293K to 773K and thereby calculate a production rate for this system.

Revision – To calculate the time it takes a mass to heat up is given by the equation.

Qnett=mc∆T

Solution:

View Factor Section

All 25 view factors must be calculated.  The calculation methodology MUST be stated:

Calculating the view factor for surface 5

Reference used: (from book [1] Table 13.4)

So , for our surfaces ,

ri = 0.5 m , rj = 1m ,L = 2 – 0.5 = 1.5 m, Ri = (0.5/1.5) = 0.33, Rj = (1/1.5) = 0.66

S = 14.18

F53 = 0.288

F51 = F54 = F55 = 0

So, F51+ F52 + F53 + F54 + F55 = 1

F52 = 0.712

| Emitting (Surface 5) | Receiving surfaces |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
|  | 0 | 0.712 | 0.288 | 0 | 0 |

Calculating the view factor for surface 3 (Using C41 from Canvas Excel sheet)

R1 = R2 = 0.5

X = 6

F31 = 0.175

As we know, A3 F35 = A5F53

So F35 = 0.072

F33 = F34 = 0

F31+ F32+ F33+ F34 + F35 = 1

F32 = 0.753

| Emitting (Surface 3) | Receiving surfaces |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
|  | 0.175 | 0.753 | 0 | 0 | 0.072 |

Calculating the view factor for surface 1 (Using C77 from canvas excel sheet to find F41 and then doing reciprocity)

F41 = 0.267(from image)

Using reciprocity,

F14 = 0.267, F13 = 0.175, F12 = 0.558

F11 = F15 = 0

| Emitting (Surface 1) | Receiving surfaces |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
|  | 0 | 0.558 | 0.175 | 0.267 | 0 |

Calculating the view factor for surface 4

F41 = 0.267(calculated preciously using C77)

F45 = F44 = F43 = 0

Using Reciprocity, F42 = 0.733

| Emitting (Surface 4) | Receiving surfaces |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
|  | 0.267 | 0.733 | 0 | 0 | 0 |

Calculating the view factor for surface 2

We can find view factor of Surface 2 only by using reciprocity:

| Emitting (Surface 2) | Receiving surfaces |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
|  | 0.139 | 0.262 | 0.188 | 0.366 | 0.0445 |

Total View Factor Table:

Emitting Surfaces

Receiving Surfaces

|  | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0.558 | 0.175 | 0.267 | 0 |
| 2 | 0.139 | 0.262 | 0.188 | 0.366 | 0.0445 |
| 3 | 0.175 | 0.753 | 0 | 0 | 0.072 |
| 4 | 0.267 | 0.733 | 0 | 0 | 0 |
| 5 | 0 | 0.712 | 0.288 | 0 | 0 |

Resistance network:

Note: Naming is just for figure I will change it accordingly.

Dimensions of surface:

| SURFACE | DESCRIPTION | AREA (m2) |
| --- | --- | --- |
| 1 | Bottom Surface | 3.1415 |
| 2 | Left side wall | 12.566 |
| 3 | Top Surface | 3.1415 |
| 4 | Billet Side | 1.5707 |
| 5 | Billet Top | 0.7853 |

Total View Factor Table:

Emitting Surfaces

Receiving Surfaces

|  | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0.558 | 0.175 | 0.267 | 0 |
| 2 | 0.139 | 0.262 | 0.188 | 0.366 | 0.0445 |
| 3 | 0.175 | 0.753 | 0 | 0 | 0.072 |
| 4 | 0.267 | 0.733 | 0 | 0 | 0 |
| 5 | 0 | 0.712 | 0.288 | 0 | 0 |

Surface temperature Table:

| Surface | Temperature / K |  /W m-2K-1 | Emissivity |
| --- | --- | --- | --- |
| 1 | 1200 | 20 | 0.8 |
| 2 | 1400 | 15 | 0.8 |
| 3 | 1600 | 20 | 1 |
| 4 | 293 | 15 | 0.5 |
| 5 | 293 | 10 | 0.5 |

σ = 5.67037 × 10-8 W.m-2.K-4

### Radiosity (J )

Radiosity is the total radiant energy leaving the surface per unit area and per unit time.

So in our case, G is irradiation and J is the radiosity leaving.

J = εσT4+ ρG

For matrix multiplication:

J – ρG =  εσT4

J – ρFijJ =  εσT4

J (1 - ρFij ) = εσT4

J = (1 - ρFij )-1 * εσT4

X = A-1 B

So radiosity,

| Radiosity | W/m2 | kW/m2 |
| --- | --- | --- |
| J1 | 135758.548 | 135.758 |
| J2 | 211238.574 | 211.238 |
| J3 | 371613.657 | 371.613 |
| J4 | 95751.658 | 95.751 |
| J5 | 128922.254 | 128.922 |

### Incoming Radiation to each surface (Gi)

The irradiation received by each surface i is:

Gi=j=15FijJj

| Irradiation (G) | W/m2 |
| --- | --- |
| G1 | 208469.206 |
| G2 | 184860.459 |
| G3 | 192102.794 |
| G4 | 191085.407 |
| G5 | 257426.597 |

### Total radiative heat surface

The net radiation leaving the surface i (qrad")

qrad,i"= Ji- Gi

- qrad,i">0 : surfaces lose radiation

- qrad,i"<0 : surface gains radiation

| Surface | qrad,i"= Ji- Gi |
| --- | --- |
| 1 | -72710.658 |
| 2 | 26378.115 |
| 3 | 179510.8630 |
| 4 | -95333.749 |
| 5 | -128504.3430 |

### Total radiative heat transfer (qrad,i)

| SURFACE | AREA, Ai (m2) | FLUX, qrad,i" (W/m2) | TOTAL qrad,i= Ai×qrad,i" (W) |
| --- | --- | --- | --- |
| 1 | 3.1415 | -72710.658 | -228420.5321 |
| 2 | 12.566 | 26378.115 | 331467.3931 |
| 3 | 3.1415 | 179510.8630 | 563933.3761 |
| 4 | 1.5707 | -95333.749 | -149740.7196 |
| 5 | 0.7853 | -128504.3430 | -100914.4606 |

### Gas Temperature in Furnace (T_gas)

Σ (h×A×T_surface) − Q_total = h×A×T_gas

So, using this formula,

Gas temperature (K)  =  1561.721441934695

### Convective heat transfer

| SURFACE | hi (W/m2K) | Ai (m2) | Ti (K) | qconv,i |
| --- | --- | --- | --- | --- |
| 1 | 20 | 3.1415 | 1200 | -22726.9 |
| 2 | 15 | 12.566 | 1400 | -30482.87 |
| 3 | 20 | 3.1415 | 1600 | 1202.520 |
| 4 | 15 | 1.5707 | 293 | -29891.71 |
| 5 | 10 | 0.7853 | 293 | -14944.904 |

### Heating time calculation and production rate

Radius of Billet = 0.5 m

Length (L) = 0.5 m

s=8000 kg/m3

cs=420 J/kg K

Volume (v)= π×r2×L = 0.3296 m2

Mass (m) = 8000 * 0.3296 = 2636.8 kg

Qnet = Q4 + Q5

Qrad,total​=−(149,740.72+100,914.46)=250,655.18 W

Qconv,total​=−(29,891.71+14,944.90)=44,836.61 W

Qnet = 295491.79 W

Time (t)  Time (t) =2636.8×420×(773-293 )295491.79 = 1799 sec

Production rate 3600 / 1799 = 2 batch per hour (approx)

Conclusion

Overall after solving this problem, it is concluded that radiation is dominant mode of heat transfer in high temperature furnace environment. The calculated value of T_gas = 1561.7 K

At this temperature the production rate of furnace is 2 batches per hour when temperature of 1561.7K is maintained.

Appendix

Code used in python to calculate the values above

import numpy as np

eps = np.array ([0.8,0.8,1,0.5,0.5])

T = np.array ([1200,1400,1600,293,293])

sigma=5.670374419e-8

F = np.array ([

[0,0.558,0.175,0.267,0], # sum= Fij
[0.139,0.262,0.188,0.366,0.0445],
[0.175,0.753,0,0,0.072],
[0.267,0.733,0,0,0],
[0,0.712,0.288,0,0]
])

a = 1-eps #(rho)

D = np.diag(a)

I=np.eye(5)

M = I - D@F

rhs=eps*sigma*(T**4)

J = np.linalg.solve(M,rhs)

print("Matrix M=\n",M)

print("\nRight hand side vector (rhs)=\n",rhs)

print("\nRadiosity vector j (W/m^2) =\n",J)

import numpy as np

F = np.array ([

[0,0.558,0.175,0.267,0], # sum= Fij
[0.139,0.262,0.188,0.366,0.0445],
[0.175,0.753,0,0,0.072],
[0.267,0.733,0,0,0],
[0,0.712,0.288,0,0]
])

J = np.array ([135758.548, 211238.574 , 371613.657, 95751.658 , 128922.254])

G= F@J

print("\nIncoming radiation G (Wlm^2) = \n",G)

import numpy as np

J = np.array ([135758.548, 211238.574 , 371613.657, 95751.658 , 128922.254])

G = np.array ([208469.206,184860.459,192102.794,191085.407,257426.597 ])

A = np.array ([3.1415,12.566,3.1415,1.5707,0.7853])

Q = A * (J-G)

print("Net radiative heat flux from each surface(W) = \\n", Q)

import numpy as np

Q = np.array([-72710.658,26378.115,179510.8630,-95333.749,-128504.343])

A = np.array ([3.1415,12.566,3.1415,1.5707,0.7853])

T = np.array ([1200,1400,1600,293,293])

H = np.array ([20, 15, 20, 15, 10])

n = np.sum(H*A*T) - np.sum(Q)

d = np.sum(H*A)

t_gas = (n/d)

print("Gas temperature (K) = ", t_gas)

import numpy as np

A = np.array ([3.1415,12.566,3.1415,1.5707,0.7853])

H = np.array ([20, 15, 10, 15, 15])

T = np.array ([1200,1400,1600,293,293])

Q = np.array([-72710.658,26378.115,179510.8630,-95333.749,-128504.343])

T_gas = 1561.721441934695

q_conv = H*A*(T-T_gas)

q_t = q_conv + Q

print("\\n Convection rate of each surface = ",q_conv)

print("\\n Total heat flux to all surface = ",q_t)

Reference:

[1] Theodore L. Bergman, Adrienne S. Lavine, Frank P. Incropera, David P. De Witt Fundamentals Of Heat And Mass Transfer, Seventh Edition John Wiley & Sons, Inc. ( 2011)